Effect of Molecular Weight of PCL on the Structure and Mechanical Properties of PCL/PET Composite Vascular Scaffold Prototype

LI Chao-Jing(李超婧), Mohammed Abedalwafa, Wang Fu-Jun(王富军)*, GE Peng(葛  鹏), CHEN Pei-feng(陈培峰), WANG Lu(王  璐)

Key Laboratory of Textile Science and Technology, Ministry of Education, Donghua University, Shanghai 201620, China

# Abstract：A new scaffold has been developed, which made from Poly-epsilon-caprolactone (PCL) membrane with porous structure, and reinforcement of PCL scaffold was achieved by embedding Polyethylene terephthalate (PET) weft-knitted tubular fabric. The aim of this paper is to study the variation tendency of the morphology and the mechanical properties of the sample with the changing of molecular weight. Weighing method was used to analyze the porosity of the sample, and SEM images were taken to observe porous structure. The tensile and compressive strengths of the samples were tested by the universal mechanical tester and radial compression apparatus, respectively. And the results showed that the porosity and compressive strength were improved when increasing the molecular weight, and the elastic recovery rate was also improved slightly. However, molecular weight has little impact on the tensile strength properties, because the PET tubular fabric provides most of the strength support rather than PCL membrane.

Key words: poly (Ɛ-caprolactone) (PCL); molecular weight; porous structure; mechanical properties; vascular scaffold

*Received date: 2013-06-12

Foundation items: Fundamental Research Funds for the Central Universities; National Natural Science Foundation of China (No. 31100682 and No. 30972942)

*. Correspondence should be addressed to Wang Fujun, E-mail: wfj@dhu.edu.cn or wanglu@dhu.edu.cn